## Screen Printing Squeegee Guide

1. What is a Squeegee?

A squeegee is a critical tool in screen printing used to push ink through a stencil (mesh screen) onto the substrate (e.g., fabric, paper, or plastic).

Key Parts:

- **Blade**: Rubber edge that transfers ink (durometer/shape varies).

- **Handle**: Holds the blade; can be wood, aluminum, or plastic.

- **Edge Angle**: Sharp (for detail) or rounded (for coverage).

---

2. Types of Squeegees**

*(Visual: Compare different squeegees side by side)*

| Type          | Best For                | Blade Hardness (Durometer) |

|---------------|-------------------------|----------------------------|

| **Standard**  | General printing        | 60-70 Shore A              |

| **Hard Blade**| High-detail prints      | 75-90 Shore A              |

| **Soft Blade**| Textile/uneven surfaces | 55-60 Shore A              |

| **Dual-Duro** | Versatile use           | Mixed hardness (e.g., 70/90)|

---

3. Choosing the Right Squeegee

**Factors to Consider:**

- **Ink Type**: Water-based (softer blade) vs. plastisol (harder blade).

- **Mesh Count**: Higher mesh = sharper blade angle.

- **Material**: Fabric (softer) vs. posters (harder).

**Pro Tip:**

- *Replace blades when worn or nicked to avoid uneven prints.*

---

4. Squeegee Techniques**

*(Visual: Show hand positioning and angles)*

- **Angle**: 45°–75° (steeper for detail, shallow for flood coats).

- **Pressure**: Consistent, firm strokes.

- **Stroke Speed**: Moderate for even ink deposit.

5. Maintenance & Longevity**

- Clean blades with ink remover after use.

- Store horizontally to avoid warping.

- Rotate blades for even wear.

**Did You Know?**

*Auto presses use pneumatic squeegees for high-volume printing!*

---

**Final Graphic Suggestion:**

- Use icons (blade types, angles, ink drops).

- Add a comparison table/image for quick reference.

- Include a "Troubleshooting" section (e.g., "Streaks? Check blade damage!").

Let me know if you’d like the content adapted for a specific format (e.g., PowerPoint, Instagram carousel)!